VTLF Acoustical Panels

Kinetics Noise Control, Inc.

PO Box 655

6300 Irelan Place

Dublin, OH 43017

Phone: (614) 889-0480

Fax: (614) 889-0075

E-mail: intsales@kineticsnoise.com

www.kineticsnoise.com

SECTION 09 84 33.11

SOUND-ABSORBING WALL PANELS

PART 1 GENERAL

  - SECTION INCLUDES

- Sound-absorbing wall panels, custom-fabricated and fabric-finished.

1.02	REFERENCES

- ASTM International

- ASTM C423 Standard Test Method for Sound Absorption and Sound Absorption Coefficients by the Reverberation Room Method.

- ASTM E795 Standard Practices for Mounting Test Specimens During Sound Absorption Tests.

1.03	SUBMITTALS

- General: Submit listed submittals in accordance with Conditions of the Contract and Division 1 Submittal Procedures Section.

- Product Data: Submit product data sheet, for specified products.

- Shop Drawings: Submit shop drawings showing layout, edge profiles and panel components, including anchorage, accessories, finish colors and textures.

- Samples: Submit selection and verification samples of finishes, colors and textures.

- Test Reports: Certified test reports showing compliance with specified performance requirements.

- Standard Systems: Submit certified copies of previous test reports substantiating performance of system in lieu of retesting.

1.04	DELIVERY, STORAGE, AND HANDLING

- General: Comply with Division 1 Product Requirements Section.

- Delivery: Deliver materials in manufacturer’s original, unopened, undamaged containers with identification labels intact.

- Storage and Protection: Store materials protected from exposure to harmful environmental conditions and at temperature and humidity conditions recommended by the manufacturer.

1.05	PROJECT CONDITIONS

- Environmental Requirements: Do not install panels until wet work, such as concrete and plastering, is complete; the building is enclosed; and the temperature and relative humidity are stabilized at 60 - 80 degrees F (16 - 27 degrees C) and 35% MINIMUM RH and 55% MAXIMUM RH, respectively.

PART 2 PRODUCTS

2.01	SOUND-ABSORBING WALL PANELS

- Manufacturer: Kinetics Noise Control, Inc.

- Contact: PO Box 655, 6300 Irelan Place, Dublin, OH 43017; Telephone: (614) 889-0480; Fax: (614) 889-0540; E-mail: intsales@kineticsnoise.com; Web site: www.kineticsnoise.com.

2.02	MANUFACTURED UNITS

- VTLF Low Frequency Absorption Panels

- Thickness:  2 1/8 inches (54 mm) 
Size: As indicated on the drawings up to a maximum 48 inch (1219 mm) x 120 inch (3048 mm) panel.

- Edge Detail: [Square] [Round] [Mitered] [Beveled] [Pencil] hardened with a Class A hardening solution.

- Facing: [100% polyester fabric, FR 701 Style 2100 by Guilford of Maine] [Factory approved customer selected fabric].  Designer selected fabrics must be approved by the panel manufacturer as acceptable quality for wrapping and covering core materials.  Some fabrics are unstable, too stiff, or lack the weight and thread density for producing an acceptable finish product.

  - Color: [As selected from panel manufacturer’s stocked range of colors] [As selected from fabric manufacturer’s full range of colors].

NOTE TO SPECIFIER – NRC sound absorption data is based on testing with Guilford FR701 fabric.  To maintain specified sound absorption, fabrics should be selected that allow air to pass easily through the material into the acoustical core.

- Acoustical Performance per ASTM C423. All 1/3 octave band sound absorption coefficients from 80 Hz to 125 Hz must exceed .40 and no frequency at 500 Hz and above can exceed .40.

- Substitutions: Any alternate for VTLF must be accompanied with testing from an certified independent testing laboratory verifying acoustical performance

- Mounting Accessories: [HS impaling clips] [Z-clips] [Velcro] [Rotofast Snap-On].

PART 3 EXECUTION

3.01	MANUFACTURER’S INSTRUCTIONS

- Compliance: Comply with manufacturer’s product data, including product technical bulletins, product catalog installation instructions and product carton instructions for installation.

3.02	EXAMINATION

- Site Verification of Conditions: Verify that substrate conditions, which have been previously installed under other sections, are acceptable for product installation in accordance with manufacturer’s instructions.

- Verify that stud spacing is 16 inches (406 mm) o.c., maximum, for panels installed over open studs.

- Do not install panels until unsatisfactory conditions are corrected.

3.03	CLEANING

- Follow manufacturer’s instructions for cleaning panels soiled during installation. Replace panels that cannot be cleaned to as new condition.

- Keep site free from accumulation of waste and debris.

END OF SECTION