MidTones Wood Diffusers

Kinetics Noise Control, Inc.

PO Box 655

6300 Irelan Place

Dublin, OH 43017

Phone: (614) 889-0480

Fax: (614) 889-0075

E-mail: intsales@kineticsnoise.com

www.kineticsnoise.com

SECTION 09 84 33.12

sound-DIFFUSING wall PANELS

GENERAL

SECTION INCLUDES

    - Sound-diffusing, custom-fabricated wood wall units (planks).

REFERENCES

    - ASTM International:

      - ASTM C423 Standard Test Method for Sound Absorption and Sound Absorption Coefficients by the Reverberation Room Method.

      - ASTM E795 Standard Practices for Mounting Test Specimens During Sound Absorption Tests.

SUBMITTALS

    - General: Submit listed submittals in accordance with Conditions of the Contract and Division 1 Submittal Procedures Section.

    - Product Data: Submit product data sheet, for specified products.

    - Shop Drawings: Submit shop drawings showing layout, edge profiles and panel components, including anchorage, accessories, finish colors and textures.

    - Samples: Submit selection and verification samples of finishes, colors and textures.
Product samples with finished wood surfaces will be provided as requested to match a product type, wood type, and a finish if requested. Wood is a natural material, with variations in color, texture, and figure. Variations in color, texture and grain patterns are to be expected.

DELIVERY, STORAGE, AND HANDLING

    - General: Comply with Division 1 Product Requirements Section.

    - Delivery: Deliver materials in manufacturer’s original, unopened, undamaged containers with identification labels intact.

    - Storage and Protection: Store materials protected from exposure to harmful environmental conditions and at temperature and humidity conditions.  Environmental conditions required for storage are the same as for installation, see 1.05 Project Conditions.

PROJECT CONDITIONS

    - Environmental Requirements: Do not install diffusers or reflectors until wet work, such as concrete and plastering, is complete; the building is enclosed; and the temperature and relative humidity are stabilized at 60 - 80 degrees F (16 - 27 degrees C) and 40% to 50%, respectively.

PRODUCTS

SOUND-DIFFUSING WOOD PLANKS

    - Manufacturer: Kinetics Noise Control, Inc.

      - Contact: PO Box 655, 6300 Irelan Place, Dublin, OH 43017; Telephone: (614) 889-0480; Fax: (614) 889-0540; E-mail: intsales@kineticsnoise.com; Web site: www.kineticsnoise.com.

    - Substitutions: No substitutions permitted.

MANUFACTURED UNITS

    - MidTones Wood Sound Diffuser Planks

      - Constructed from 3/8 inch (9.5 mm) thick hardwood, all pieces shall be assembled to create an 18 inch (457 mm) wide solid wood panel oriented as shown on the drawings.  Panel lengths shall be as shown on the drawings and no greater than 96 inches (2438 mm).  Select [maple] [birch] [oak] [poplar] [American cherry] or   other approved wood species.  Optional:  [Trim pieces for closing out the open ends of the diffusers shall be provided as shown on drawings.  Trim will match the wood type and finish of the diffuser].

      - Finish:  Select [stained with clear coat] [clear coat only] [unfinished]

      - Planks are designed and constructed to produce equal width wells of varying depths based on reflection phase grating principles.  The well depths shall be determined by a quadratic residue theory sequence to optimize sound diffusion.  There shall be a sequence of 7 elevations across the face.

      - Sound Diffusion:  Diffusion effects shall be based on test results for a small scale panel utilizing the same sequence of well depths and quadratic residue theory.  Results shall be derived as estimates based on the small scale testing.  Testing of the reduced size model shall be in compliance with AES-4id-2001.  The Diffusion Coefficients shall be no less than the following 500 Hz = 0.67, 630 Hz = 0.71, 800 Hz = 0.73, 1000 Hz = 0.76, 1250 Hz = 0.76, 1600 Hz = 0.79, and 2000 Hz = 0.79.

      - Mounting: Planks are installed flush to the wall or ceiling surface per manufacturer’s guidelines. Planks installed with wells oriented lengthwise in the vertical direction will diffuse sound in the horizontal plane.  Planks installed with wells oriented lengthwise in the horizontal direction will diffuse sound in the vertical plane.

EXECUTION

NOTE TO SPECIFIERS – Paragraph below is an addition to CSI SectionFormat and a supplement to this specification. Retain or delete paragraph below per project requirements and specifier’s practice.

MANUFACTURER’S INSTRUCTIONS

    - Compliance: Comply with manufacturer’s product data, including product technical bulletins, product catalog installation instructions and product carton instructions for installation.

EXAMINATION

    - Site Verification of Conditions: Verify that substrate or supporting structure, which has been previously installed under other sections, is acceptable for product installation in accordance with manufacturer’s instructions.

CLEANING

    - Follow manufacturer’s instructions for cleaning planks soiled during installation. Replace planks that cannot be cleaned to as new condition.

    - Keep site free from accumulation of waste and debris.

END OF SECTION