QUIETTILE Noise-Control Ceiling Tile System

Kinetics Noise Control, Inc.

PO Box 655

6300 Irelan Place

Dublin, OH 43017

Phone: (614) 889-0480

Fax: (614) 889-0075

E-mail: intsales@kineticsnoise.com

www.kineticsnoise.com

SECTION 09 51 23

ACOUSTICAL TILE CEILINGS

- GENERAL

  - SECTION INCLUDES

Noise-control ceiling tiles, custom-fabricated and ACT-finished.

  - REFERENCES

    - ASTM International

ASTM E1414 Standard Test Method for Airborne Sound Attenuation Between Rooms Sharing a Common Ceiling Plenum.

      - ASTM C423 Standard Test Method for Sound Absorption and Sound Absorption Coefficients by the Reverberation Room Method.

      - ASTM E84 Standard Test Method for Surface Burning Characteristics of Building Materials.

  - SUBMITTALS

    - Submit under provisions of Section 01 30 00.

    - Product Data:  Manufacturer's data sheets on each product to be used, including:

Preparation instructions and recommendations.

      - Storage and handling requirements and recommendations.

  - QUALITY ASSURANCE

    - Product Qualifications: Products shall meet the following criteria:

Surface Burning Characteristics (ASTM E84)

Flamespread: 25, maximum.

        - Smoke Developed: 450, maximum.

  - DELIVERY, STORAGE, AND HANDLING

    - Store products in manufacturer's unopened packaging until ready for installation.

    - Store and dispose of solvent-based materials, and materials used with solvent-based materials, in accordance with requirements of local authorities having jurisdiction.

  - PROJECT CONDITIONS

    - Maintain environmental conditions (temperature, humidity, and ventilation) within limits recommended by manufacturer for optimum results.  Do not install products under environmental conditions outside manufacturer's absolute limits.

PART 2 PRODUCTS

  - Noise-Control Ceiling Tiles

    - Manufacturer: Kinetics Noise Control, Inc.

Contact: PO Box 655, 6300 Irelan Place, Dublin, OH 43017; Telephone: 614-889-0480; Fax: 614-889-0540; Email: intsales@kineticsnoise.com; Web site:  www.kineticsnoise.com

Substitutions:  Not permitted.

Requests for substitutions will be considered in accordance with provisions of Section 01 60 00.

  - MANUFACTURED UNITS

    - QuietTile

Thickness: Overall panel thickness shall be determined by the facing tile specified. Additional thickness of the Sound Deadener (damping) and mass layer bonded to the facing tile shall be 11/16 inches (17.5 mm).

Size: [24 inches x 24 inches (610mm x 610 mm)] [24 inches x 48 inches (610mm x 1220 mm)].

Edge Detail: Based on facing tile selected by Architect.

Ceiling facing tile as selected by Architect.

Core: Class A fire rated mass layer weighing 1.80 lb/sf (8.8 kg/sm) and a 1/16 inch (2 mm) sound deadening asphaltic mastic layer bonded to the facing tile and supplied by the manufacturer.

Acoustical Performance: Per ASTM E1414, Ceiling Attenuation Class (CAC) of 49 minimum, or CAC 51 with 3 pcf, 1” fiberglass board added above the tile.  Per ASTM C423, Type E400 mounting: Noise Reduction Coefficient (NRC) minimum 0.60 when tested with mineral fiber facing tile. Product was tested with Armstrong Cortega Acoustical Ceiling Tile in an independent lab.

NOTE TO SPECIFIER – Products listed below are optional.

LightHood Fixture Covers

Quantity: Each light fixture shall require a Fixture Cover

Size: [24 inches x 24 inches (610mm x 610 mm)] [24 inches x 48 inches (610mm x 1220 mm)].

Acoustical Performance:  As tested per ASTM E2611TL minimums are as followed: 500 Hz-9 dB, 1,000 Hz-10 dB, 2,000 Hz- 12 dB, 4,000 Hz- 16 dB.  Testing must be conducted by an independent lab.

Fiberglass Board- 3 PCF fiberglass board

Size: [24 inches x 24 inches (610mm x 610 mm)] [24 inches x 48 inches (610mm x 1220 mm)].

PART 3 EXECUTION

  - EXAMINATION

    - Do not begin installation until substrates have been properly prepared.

If substrate preparation is the responsibility of another installer, notify Architect of unsatisfactory preparation before proceeding.

- 3.02	PREPARATION

    - Clean surfaces thoroughly prior to installation.

Prepare surfaces using the methods recommended by the manufacturer for achieving the best result for the substrate under the project conditions.

  - INSTALLATION

    - Install in accordance with manufacturer's instructions.

  - PROTECTION

    - Protect installed products until completion of project.

    - Touch-up, repair or replace damaged products before Substantial Completion.

END OF SECTION