SCATTERBOX Diffuser

Kinetics Noise Control, Inc.

PO Box 655

6300 Irelan Place

Dublin, OH 43017

Phone: (614) 889-0480

Fax: (614) 889-0075

E-mail: intsales@kineticsnoise.com

www.kineticsnoise.com

SECTION 09 84 33.12

SOUND-DIFFUSING WALL PANELS

GENERAL

SECTION INCLUDES

    - Sound-diffusing wall panels.

SUBMITTALS

    - General: Submit listed submittals in accordance with Conditions of the Contract and Division 1 Submittal Procedures Section.

    - Product Data: Submit product data sheet, for specified products.

    - Shop Drawings: Submit shop drawings showing layout, edge profiles and panel components, including anchorage, accessories, finish colors and textures.

    - Samples: Submit selection and verification samples of finishes, colors and textures.
Product samples with finished wood surfaces will be provided as requested to match a product type, wood type, and a finish if requested. Wood is a natural material, with variations in color, texture, and figure. Variations in color, texture and grain patterns are to be expected.

DELIVERY, STORAGE, AND HANDLING

    - General: Comply with Division 1 Product Requirements Section.

    - Delivery: Deliver materials in manufacturer’s original, unopened, undamaged containers with identification labels intact.

    - Storage and Protection: Store materials protected from exposure to harmful environmental conditions and at temperature and humidity conditions.  Environmental conditions required for storage are the same as for installation, see 1.04 Project Conditions.

PROJECT CONDITIONS

    - Environmental Requirements: Do not install diffusers or reflectors until wet work, such as concrete and plastering, is complete; the building is enclosed; and the temperature and relative humidity are stabilized at 60 - 80 degrees F (16 - 27 degrees C) and 40% to 50%, respectively.

PRODUCTS

SOUND-DIFFUSING WALL PANELS

    - Manufacturer: Kinetics Noise Control, Inc.

      - Contact: PO Box 655, 6300 Irelan Place, Dublin, OH 43017; Telephone: (614) 889-0480; Fax: (614) 889-0540; E-mail: intsales@kineticsnoise.com; Web site: www.kineticsnoise.com.

    - Substitutions: No substitutions permitted.

MANUFACTURED UNITS

    - ScatterBox Wood Diffuser Panels

      - Panels shall be constructed from solid hardwood, 3/8-inch (10 mm) thick perimeter frame and ¼ inch (6 mm) thick interior ribs.  Panel dimensions 23-5/8 inches (600 mm) x 23-5/8 inches (600 mm) x 3 inches (76 mm).

      - The panel shall have 64 wells of 4 different depths arranged in a quasi – random pattern to optimize diffusion and minimize visual pattering.

      - The well array design and varying depths shall create sound diffusion in two dimensions, both vertical and horizontal (wall mounted).

      - Panels are wall mounted per manufacturer’s guidelines. Panel dimensions also allow for installation in a standard lay-in ceiling grid.

EXECUTION

NOTE TO SPECIFIER: Paragraph below is an addition to CSI SectionFormat and a supplement to this specification. Retain or delete paragraph below per project requirements and specifier’s practice.

MANUFACTURER’S INSTRUCTIONS

    - Compliance: Comply with manufacturer’s product data, including product technical bulletins, product catalog installation instructions and product carton instructions for installation.

EXAMINATION

    - Site Verification of Conditions: Verify that substrate or supporting structure, which has been previously installed under other sections, is acceptable for product installation in accordance with manufacturer’s instructions.

CLEANING

    - Follow manufacturer’s instructions for cleaning panels soiled during installation. Replace panels that cannot be cleaned to as new condition.

    - Keep site free from accumulation of waste and debris.

END OF SECTION